| BLAR mHOR OPERATIONS OPERATIONAL PROCEDURE (OPP) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Title |  |  |  | Version Number |
| OPERATIONS-OPP-EFFLUENT PLANT CHEMICAL IN USE |  |  |  | 1 |
| Revised or amended date | 01/08/2022 | Next revision date |  | 01/08/2023 |
| The instruction within this document is a guide only and should be an accompaniment to practical training |  |  |  |  |
| This document shall only be amended by designated persons on the authorisation of the Processing Departmental Manager |  |  |  |  |

Purpose: To detail all cleaning chemicals currently permitted to be used within the effluent plant

Responsibility: Operations Manager, Operations Line Management, Effluent Plant Operators operatives

Health and Safety: Chemical Handling training

References: PRO-CHEMICAL CONTROL

Product name:; Type:; Use/ dosage%; Recommended contact time; Application and sampling method. ANTIFOAM FDP; Defoamer for Industrial applications. Defomer for food and drink applications.; 1-2 l/h; N/A; Pump Dosing. AROMA LIQUID; Perfumes, fragrances; 1-10%; N/A; Power washer dosing pump. Deodorising Scrubber. Diluting in Designated Bin. FERRIC CHLORIDE SOLUTION; Coagulant and flocculant.; 12-28 l/h; N/A; Pump dosing into the effluent treatment plant.. SODIUM HYDROXIDE 30-40%; Neutralizer; 12-22l/h; N/A; Pump dosing into the effluent treatment plant.. SODIUM HYPOCHLORITE 5-14%; 10-15l/h; N/A; Pump dosing into the effluent treatment plant.. FORMIC ACID 85%; To keep low ph in product (VISCERA); 300l/26T tanker; N/A; Pump dosing into the road tanker. FORMIC ACID 85%; To keep low ph in product (SCREED OFF MATERIALS)); As needed.; N/A; Pump dosing into the storage tank.