| BLAR mHOR OPERATIONS STANDARD OPERATING PROCEDURE(SOP) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Title |  |  |  | Version Number |
| OPERATIONS-SOP-HARVESTING-REMOVAL OF DISPATCHED/TRAPPED FISH FROM RB6-RB7 MACHINES |  |  |  | 6 |
| Revised or amended date | 01/08/2022 | Next revision date |  | 01/08/2023 |
| The instruction within this document is a guide only and should be an accompaniment to practical training |  |  |  |  |
| This document shall only be amended by designated persons on the authorisation of the Processing Departmental Manager |  |  |  |  |

Purpose: This operating procedure covers the safe removal of dispatched / trapped fish from the RB6-RB7 Stunning, bleeding machines

Responsibility: All personnel

Health and Safety: Protective clothing, Protective footwear, Protective gloves (where required), Safety goggles (where required)

References: RA

| Task steps: | Precautions / considerations: |
| --- | --- |
| - Eye Check & inform | - Always complete an Eye-Check Risk Assessment! - Ensure all personnel are aware the process taking place |
| - Hazard Identification | Pneumatically controlled Stunner / bleeding machines Models RB6 – RB7 pose a significant risk to the operator if the correct isolation procedures are not followed prior to manually extracting dispatched, trapped fish from the blade – hammer mechanisms |
| - Warning signage & Safety measures | Bleeding blade Warning signage - Danger – Sharp edges watch your fingers - Warning – Remove bleeder cylinder before starting any work on machine - Caution – Heavy (45kg) Must be lifted by two people Note: Only trained experienced machine minders will operate this machinery. Only trained experienced service engineers will service, adjust and maintain this machinery Both the RB6 – RB7 machines models must have their HAMMER & KNIFE mechanisms isolated before a manual extraction of trapped fish is attempted. Under no circumstances must the manual extraction of trapped fish take place without the isolation of pneumatic air supply to the Hammer & Knife mechanisms, failure to follow this procedure could lead to serious injury. |
| - RB6-RB7 Machine models | RB6 Model operates with airflow controls & valves RB7 Model (later revised model) operates utilizing solenoids and PLC control |
| - Resetting the machine via the auto reset buttons | ### Under normal circumstances pressing the automatic reset button of either the RB6-RB7 ### machines will retract the hammer and the blade mechanisms, opens the FAD fish ### alignment device and opens the trigger plate, releasing trapped fish Should a fish become trapped in the machine mechanism for any reason your first course of action will be to: - Press the Automatic Reset Button on both the RB6 / RB7 Models - Machines will reset retracting blade and hammer mechanisms allowing trapped fish to pass through the machine - The RB6 machine also has a reset button accessible under the machine hood via the access cover, press this to reset the machine Reset Button RB6 / RB7 Models Reset button (RB6 only) Hood access - At this stage it is not necessary to isolate the pneumatic air supply - Only if the auto reset procedure fails to release trapped fish should you undertake a manual extraction At no time put your hand / hands into the knife / hammer mechanism area unless the machines pneumatics have been isolated and system has been given time to exhaust |
| - Pneumatic isolation in order to manually extract trapped fish | If the automatic reset procedure for the RB6 / RB7 machines fails to release trapped fish a manual extraction must be performed. Until the Pneumatics have been fully isolated and the machine given time to fully exhaust the machine can be presumed live and therefore a serious risk of injury from the knife and hammer remains Isolating the pneumatics for RB6 / RB7 machines The air supply to the machine must be isolated in order as described below - Isolate Knife - RED Airline - Isolate Hammer - Black airline - Isolate Air Control - Grey airline - Allow the airline to fully exhaust before proceeding ( you may be able to hear the air exhausting, this will take few seconds) Note: Blue lines are extraction lines for removing ozone’s only - When turning the machine back on repeat the same procedure in reverse order Red = Knife feed Black = Hammer feed Grey = Control feed |
| - Opening the machine to access hammer cylinder | Opening the machine hood will fully exhaust the airlines and expose the hammer cylinder - Release and lift RB6 / RB7 machine hood cover - Once opened any residual air will be exhausted (this may take a few seconds) - The hammer cylinder is now ready to be lifted Note: you should not put your hand / hands into the trigger, knife or hammer zone at any point |
| - Lifting the hammer cylinder | Once exposed you can now access and lift the hammer cylinder allowing the safe manual removal of trapped fish - Place hand on top of the cylinder and lift, if there is resistance, wait a few seconds there may still be air that requires exhausting from the system - Once all air has been exhausted, the cylinder will lift freely and trapped fish can be safely removed Note: take care to keep hand / hands clear of knife area at all times Once the trapped fish has been safely removed the machine can be reinstated - Drop the hammer cylinder back into position - Close the machine cover and latch in place - Turn on Air control = Grey - Turn on Hamer = Black - Turn on Knife = Red - Press reset button - Remember failure to follow the safe operations procedure puts the operator at serious risk of injury - Never put hand / hands into the Blade mechanism – hammer mechanism area of this machine - Ensure the machine has been isolated and all air fully exhausted before opening the machine and performing a manual extraction of a trapped fish |

| Ergonomic Risk Factors | Explanation and mitigation techniques |
| --- | --- |
| Musculoskeletal strains, sprain type injuries | - Musculoskeletal injuries due to incorrect manual handling techniques - Tissue bone damage from exposure to sharp knife blade - Use correct manual handling techniques when lifting heavy objects - Correct safe operating procedure to be used at all times - Appropriate PPE must be worn |

Note: If you come across any hazards not covered in this document, contact the Mowi Scotland Health and Safety Department to assist with developing safety measures and controls

Site Notes: ________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

| ACTIVITY | PERSON AT RISK | SIGNIFICANT HAZARDS | RISK * |  |  | RISK CONTROL MEASURES | RESIDUAL RISK ** |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | L | S | DR |  | L | S | DR |
| Removal of trapped fish from Pneumatic stunner mechanism | Machine minders & engineers | Risk of serious injury from RB6 – RB7 Machines Pneumatic Hammer Pneumatic Knife Failure to isolate Air supply to: Knife mechanism Hammer mechanism Air control | 4 | 5 | 20 | - Never insert your Hand / hands into the machine to retrieve / extract trapped fish - Ensure correct warning signage / labelling is present and clear on all machinery - All involved to read and sign Safe operating procedure - Safe operating procedure must be followed when extracting trapped fish from mechanism - Only Machine minders / engineers to perform task - Isolate air supply to knife - Isolate air supply to hammer - Isolate air control - Ensure all air has been exhausted before opening machine cover - Wait a few seconds once machine cover is opened to ensure all air is fully exhausted | 2 | 5 | 10 |
| Removal of trapped fish from Pneumatic stunner mechanism | Machine minders & engineers | Risk of serious injury from RB6 – RB7 Machines Failure to isolate air supply to the knife mechanism, hammer mechanism and air control in the correct order can leave blade in an exposed position | 3 | 5 | 15 | - Never insert your Hand / hands into the machine to retrieve / extract trapped fish - Isolation of air supply to cylinders must be carried out in correct order - Isolate air supply to knife - Isolate air supply to hammer - Isolate air control - Reverse air isolation order when reinstating machine | 2 | 5 | 10 |